# Network Management - Units II & III: Complete Study Guide

Based on Mani Subramanian, Network Management: Principles and Practice (primary reference). Covers SNMPv1, SNMPv2, RMON and ATM network management.

Note: I could not see your PPTs (none were attached). This guide follows the syllabus order you pasted and Subramanian's chapter structure (Ch. 4, 5, 8, 9, 13 of the book). If a PPT uses different figure names or case-study names, keep my explanation but use your slide's labels in the exam.

## How to use this guide

- Read Part A (prerequisites) once. Everything later builds on it.

- Read Units II and III in order. Each topic ends with Exam Points (what to write for marks).

- Finish with Part E: answer-writing templates, numericals, and a question bank.

Syllabus map

| Part | Content |
| --- | --- |
| A | Prerequisites: networks, protocols, ASN.1, management basics |
| B | Unit II (a): SNMPv1 - history, models, SMI, MIB, protocol, functional model |
| C | Unit II (b): SNMPv2 - changes, architecture, SMI, MIB, protocol, compatibility |
| D | Unit III: RMON (RMON1) and ATM network management |
| E | Exam preparation: templates, numericals, question bank |

# PART A - PREREQUISITES

## A1. Network and protocol basics

Network: a collection of nodes (hosts, routers, switches, hubs) connected by links so that they can exchange data.

Protocol: an agreed set of rules (message formats, order of exchange, error handling) that lets two entities communicate.

Layered model (TCP/IP, 4 layers) - the one SNMP lives in

Application   SNMP, HTTP, FTP, DNS      (SNMP is here)
Transport     TCP, UDP                  (SNMP uses UDP)
Internet      IP, ICMP                  (ping uses ICMP)
Network access  Ethernet, ATM, etc.

Key facts you must remember

- IP gives best-effort, connectionless delivery of packets between hosts.

- ICMP carries error/diagnostic messages (echo request/reply = ping, time exceeded = used by traceroute).

- UDP is connectionless and unreliable but light. TCP is connection-oriented and reliable but heavy.

- Why SNMP uses UDP: management traffic must work even when the network is sick. UDP has no connection set-up, so a manager can still query agents on a congested or partly failed network. Also, agents (small devices) stay simple. Cost: SNMP must handle lost messages itself (timeouts and retries by the manager).

- Well-known ports: UDP 161 (agent receives requests), UDP 162 (manager receives traps).

Client-server model: here the manager is the client (sends requests) and the agent is the server (replies). Traps are the exception: agent initiates.

## A2. Network management fundamentals

Network management = monitoring, testing, configuring, analysing, evaluating and controlling network and element resources to meet real-time, operational and QoS requirements at reasonable cost.

Five functional areas (FCAPS, ISO model)

| Area | Meaning | Example |
| --- | --- | --- |
| Fault | Detect, isolate, correct failures | Link down trap |
| Configuration | Set up and track device settings | Changing an interface state |
| Accounting | Track usage for billing/quotas | Bytes per user |
| Performance | Throughput, utilisation, response time | ifInOctets-based utilisation |
| Security | Control access, protect resources | Community string, access policy |

Basic architecture (the picture behind all of SNMP)

+-------------------+           management protocol          +------------------+
| Network           |  <------------------------------->     | Managed element  |
| Management System |    (requests / responses / traps)      |  +-------------+ |
| (NMS)  = MANAGER  |                                        |  |   AGENT     | |
|   - applications  |                                        |  |   MIB       | |
|   - MIB view      |                                        |  +-------------+ |
+-------------------+                                        +------------------+

- Manager (NMS): the software on a workstation that issues queries, receives responses and traps, and displays/analyses data.

- Agent: software in the managed device that collects data about the device and answers the manager. It also sends unsolicited alarms (traps).

- Managed object: a characteristic of a resource that can be managed, e.g. number of packets received on an interface.

- MIB (Management Information Base): the virtual database of managed objects. It is virtual because it is not stored as one physical file; each object is read from device counters/state when needed.

- Management protocol: SNMP - how manager and agent talk.

- SMI (Structure of Management Information): the rules for defining objects and naming them.

Management information in four models (Subramanian's framework)

| Model | Question it answers | In SNMP |
| --- | --- | --- |
| Organization | Who are the players, how are they arranged? | Manager, agent, proxy, 2-tier/3-tier |
| Information | How is management data defined and named? | SMI + MIB |
| Communication | How is data transported? | SNMP protocol, PDUs, UDP |
| Functional | What do users do with it (FCAPS)? | Applications, operations |

This four-model framework is the skeleton of the whole of Unit II. Many exam questions are just 'explain the SNMP model' - answer with these four.

## A3. ASN.1 basics (needed for SMI and MIB)

ASN.1 (Abstract Syntax Notation One) is a language to describe data types independently of machine representation. Because a manager on one platform and an agent on another may store integers differently, both agree on:

- Abstract syntax: the description of the data (ASN.1 types).

- Transfer syntax: how it is encoded on the wire - BER (Basic Encoding Rules).

Types

| Category | Examples |
| --- | --- |
| Primitive (simple) | INTEGER, OCTET STRING, OBJECT IDENTIFIER, NULL |
| Constructed (structured) | SEQUENCE (ordered list of different types, like a struct), SEQUENCE OF (list of same type, like an array/table) |
| Tagged | Application-specific types, e.g. IpAddress, Counter |

BER encoding: Tag-Length-Value (TLV)

+-----+--------+---------------+
| Tag | Length |    Value      |
+-----+--------+---------------+
 type    size     actual data

Example: INTEGER 5 -> tag 02, length 01, value 05 -> bytes 02 01 05.

Macro: a template used to define a new kind of object. SNMP uses the OBJECT-TYPE macro to define every managed object (see B4).

Object identifier (OID) and the global naming tree

Every managed object has a unique, globally registered name: a sequence of integers from the root of a tree.

root
        /          |          \
   ccitt(0)      iso(1)     joint-iso-ccitt(2)
                   |
                 org(3)
                   |
                 dod(6)
                   |
               internet(1)            = 1.3.6.1
   +----------+-----+--------+------------+
 directory(1) mgmt(2) experimental(3) private(4)
               |                         |
            mib-2(1)                 enterprises(1)
               |                         |
     system(1) interfaces(2) at(3)   cisco(9), ibm(2) ...
     ip(4) icmp(5) tcp(6) udp(7) egp(8) transmission(10) snmp(11)

- internet = 1.3.6.1 (iso.org.dod.internet).

- mgmt = 1.3.6.1.2; mib-2 = 1.3.6.1.2.1. All standard MIB-II objects are under here.

- private.enterprises = 1.3.6.1.4.1: vendors register their own subtrees (e.g. Cisco = 9).

- experimental = 1.3.6.1.3: used for objects under development.

Example: sysDescr = 1.3.6.1.2.1.1.1 (system group is 1, sysDescr is the first object in it).

# PART B - UNIT II (a): SNMPv1 NETWORK MANAGEMENT

## B1. Managed Network: Case Histories and Examples

Subramanian opens the SNMP chapter with real network scenarios to show why a standard management protocol is needed. Know the pattern of each case; your PPT may name specific cases, so match names.

Case type 1 - Problem isolation with basic tools A user reports 'network is slow / cannot reach a server'. The administrator uses simple tools:

- ping (ICMP echo) to check reachability and round-trip time.

- traceroute (IP TTL and ICMP time-exceeded) to find the hop where packets stop or slow down. These show that something is wrong and where, but not why.

Case type 2 - Using SNMP/MIB data to find the cause Once the suspect router/interface is found, the manager reads MIB-II objects (ifOperStatus, ifInErrors, ifInDiscards, ifOutQLen, ifInOctets) to see whether the link is down, has errors, or is congested. This is the step that needs SNMP.

Case type 3 - Performance monitoring and traffic statistics Periodic polling of interface counters, then graphing traffic (tools such as MRTG do this) to see utilisation trends and plan capacity.

Case type 4 - Remote monitoring of LAN segments A probe (RMON) at a remote site gathers statistics so that the central manager does not have to poll every device. This leads into Unit III.

Lessons (write these as conclusion)

- Networks contain equipment from many vendors, so a common, vendor-independent protocol is needed.

- Management must be simple, because agents run in low-cost devices.

- Both polling (manager asks) and event reporting (agent traps) are needed.

- Management data must be standardised and named uniformly (SMI + MIB).

Examples of management tools mentioned: ping, traceroute, tcpdump/Ethereal (packet capture), MRTG (traffic graphing), and commercial NMS platforms (HP OpenView, IBM NetView, etc.).

Exam Points: Give one case end-to-end: symptom -> ping/traceroute -> SNMP query -> cause -> fix. State why SNMP was needed.

## B2. The History of SNMP Management

Timeline (memorise years and RFC numbers - frequent short-answer question):

| Year | Event |
| --- | --- |
| 1970s | ARPANET; ICMP and ping used for basic diagnostics |
| 1987 | SGMP (Simple Gateway Monitoring Protocol) for managing Internet routers; HEMS (High-level Entity Management System) also proposed |
| 1988 | Internet Activities Board (IAB) recommends: use SNMP as the short-term solution, CMOT (CMIP over TCP/IP, OSI-based) as the long-term one. Short-term won; CMOT faded. |
| 1988-1990 | SNMPv1 RFCs 1065/1066/1067 revised as RFC 1155 (SMI), RFC 1156 (MIB-I), RFC 1157 (SNMP) in 1990 |
| 1991 | MIB-II: RFC 1213; RMON: RFC 1271 (later RFC 1757, then 2819) |
| 1993 | SNMPv2 (RFC 1441-1452), security-based (party-based) |
| 1996 | SNMPv2c (community-based SNMPv2, RFC 1901-1910) because the party-based security was too complex |
| 1998 onwards | SNMPv3 (RFC 2271-2275, later 3411-3418) with strong security |

Why SNMP won over CMIP: it was simple, quick to implement, small footprint in devices, ran over the already-deployed UDP/IP, and needed little agent memory. CMIP/CMOT was powerful but complex and resource-heavy.

Design goals of SNMP: (1) keep agent software cost and complexity minimal; (2) be extensible for future needs; (3) be independent of the architecture and mechanisms of particular hosts or gateways. SNMP = Simple Network Management Protocol.

## B3. The SNMP Model

SNMP management is described using four models.

### B3.1 Organization Model

Components

- Network Management System (NMS)/Manager: polls agents, receives traps, runs management applications.

- Agent (management agent): resident in the managed element; responds to manager requests and sends traps.

- Managed objects / MIB: the information exchanged.

Two-tier model (basic):

Manager  <----SNMP---->  Agent (managed object, e.g. router, hub)

Three-tier model (with intermediate manager/proxy):

Manager  <--SNMP-->  Remote Monitor / Proxy / Mid-level Manager  <---> Agents / Non-SNMP devices

The middle tier may be:

- RMON probe: a remote agent that collects statistics locally and passes summaries upstream (reduces polling traffic and overcomes unreachable-network problems).

- Proxy server: an SNMP agent that translates between SNMP and another management protocol (or a non-SNMP device) so that legacy/proprietary devices can be managed by an SNMP manager.

Dual role entity: the same system can act as manager to subordinate agents and as agent to a higher-level manager (manager of managers / mid-level manager).

Key points: manager-agent is a logical relationship, not a physical one; one manager can manage many agents, and one agent can be queried by several managers; the manager has the intelligence, the agent stays simple.

### B3.2 System Overview

+----------------------------------------------+           +---------------------------------+
| MANAGER (NMS)                                |           | AGENT                           |
|  Management applications / operator          |           |  Agent process                  |
|  SNMP manager entity   -- get/set requests ->|-- UDP --->|  SNMP agent entity              |
|  (UDP/IP)              <- responses, traps --|<-- IP ----|  (UDP/IP)                       |
+----------------------------------------------+           |  MIB (managed objects)          |
                                                           +---------------------------------+

The SNMP management system consists of: (1) the management information model (SMI + MIB), (2) the SNMP protocol, (3) the manager and agent applications, with a community/administrative model controlling access. SNMP operations are requests from manager, responses from agent and notifications (traps) from agent.

Exam Points: Draw the manager-agent diagram, label SNMP messages (get, get-next, set, response, trap), UDP ports 161/162, and the MIB inside the agent.

## B4. The Information Model

The information model has two parts: the SMI (rules to define objects) and the MIB (the actual objects).

### B4.1 Structure of Management Information (SMI, RFC 1155)

Purpose: define a precise, uniform way to name, describe and encode managed objects.

SMI specifies three things:

- Naming - objects get an OBJECT IDENTIFIER (OID) from the global tree (see A3).

- Syntax - each object has an ASN.1-based type.

- Encoding - BER (TLV).

A managed object is defined with five attributes

| Attribute | Meaning |
| --- | --- |
| Object name | OID, e.g. sysDescr = 1.3.6.1.2.1.1.1 |
| Syntax | Data type, e.g. DisplayString, Counter |
| Definition | Textual description of the object |
| Access | read-only, read-write, write-only, not-accessible |
| Status | mandatory, optional, obsolete (v1) |

OBJECT-TYPE macro (template) - example

sysDescr OBJECT-TYPE
    SYNTAX      DisplayString (SIZE (0..255))
    ACCESS      read-only
    STATUS      mandatory
    DESCRIPTION "A textual description of the entity."
    ::= { system 1 }

### B4.2 SNMPv1 data types

Primitive (universal):

- INTEGER - signed integer (optionally enumerated, e.g. ifOperStatus up(1), down(2), testing(3)).

- OCTET STRING - string of bytes (e.g. DisplayString, physical address).

- OBJECT IDENTIFIER - the tree name.

- NULL - placeholder (used in the value field of a get-request).

Constructed:

- SEQUENCE - defines a conceptual row of a table (list of columns of different types).

- SEQUENCE OF - defines the table itself (list of identical rows).

Defined (application-wide) types:

| Type | Description | Range / behaviour |
| --- | --- | --- |
| NetworkAddress | Choice of address families (only IpAddress in practice) |  |
| IpAddress | 4-byte IPv4 address | OCTET STRING (SIZE 4) |
| Counter | Non-negative integer that only increases, then wraps to 0 | 0 to 2^32 - 1 (4,294,967,295) |
| Gauge | Non-negative integer that can go up or down; latches (stays) at maximum, does not wrap | 0 to 2^32 - 1; e.g. current queue length |
| TimeTicks | Time in hundredths of a second since some epoch | 0 to 2^32 - 1; e.g. sysUpTime |
| Opaque | Arbitrary data wrapped as OCTET STRING (for passing data not defined in SMI) |  |

Counter vs Gauge (very common question)

| Counter | Gauge |
| --- | --- |
| Monotonically increasing | Can increase or decrease |
| Wraps to zero at 2^32 | Stops (latches) at max |
| Meaningful only as the difference between two readings | Meaningful as an absolute value |
| Example: ifInOctets, ipInReceives | Example: ifSpeed, tcpCurrEstab |

### B4.3 Names, tables and instances

Aggregate (tabular) objects

ipAddrTable      OBJECT-TYPE   SYNTAX SEQUENCE OF IpAddrEntry   (the table)
ipAddrEntry      OBJECT-TYPE   SYNTAX IpAddrEntry               (a row)
IpAddrEntry ::= SEQUENCE { ipAdEntAddr IpAddress, ipAdEntIfIndex INTEGER, ... }  (the columns)

- Only leaf (columnar) objects can be read; table and entry nodes are not-accessible.

- A scalar object's instance is the OID with .0 appended: sysDescr.0 = 1.3.6.1.2.1.1.1.0.

- A table cell's instance is column OID + index: e.g. ifDescr for interface 3 = 1.3.6.1.2.1.2.2.1.2.3 (ifIndex is the index).

- For IP-indexed tables, the index is the IP address: ipAdEntNetMask.192.168.1.5.

Lexicographic ordering: objects are ordered by comparing OID integers left to right. A table is traversed column by column, row by row within a column (all rows of column 1, then column 2 ...). GetNextRequest relies on this ordering, which lets the manager walk a table without knowing its size.

Exam Points: Define SMI; list five attributes; draw the OID tree up to mib-2; differentiate Counter/Gauge; explain scalar vs tabular instance naming with an example OID.

## B5. The SNMP Management Information Base (MIB-II)

The MIB is the collection of managed objects, organised as a tree under mib-2 (1.3.6.1.2.1). MIB-I (RFC 1156) had 114 objects; MIB-II (RFC 1213) extended it (171 objects) and is the standard for SNMPv1. Objects are grouped by function so that an agent implements whole groups (mandatory if the device supports the function).

| Group | OID | What it holds | Examples |
| --- | --- | --- | --- |
| system | mib-2.1 | General info about the device | sysDescr, sysObjectID, sysUpTime, sysContact, sysName, sysLocation, sysServices |
| interfaces | mib-2.2 | Network interfaces (a table) | ifNumber, ifTable (ifIndex, ifDescr, ifType, ifMtu, ifSpeed, ifPhysAddress, ifAdminStatus, ifOperStatus, ifInOctets, ifInErrors, ifOutOctets ...) |
| at (address translation) | mib-2.3 | IP-to-physical address map (ARP); deprecated in MIB-II, moved into ip group | atPhysAddress |
| ip | mib-2.4 | IP layer | ipForwarding, ipInReceives, ipInDiscards, ipAddrTable, ipRouteTable |
| icmp | mib-2.5 | ICMP message counts | icmpInEchos, icmpOutDestUnreachs |
| tcp | mib-2.6 | TCP layer, connection table | tcpRtoAlgorithm, tcpCurrEstab, tcpConnTable |
| udp | mib-2.7 | UDP layer | udpInDatagrams, udpNoPorts, udpTable |
| egp | mib-2.8 | Exterior Gateway Protocol | egpNeighTable |
| transmission | mib-2.10 | Media-specific (Ethernet, token ring, etc.) | dot3, dot5 |
| snmp | mib-2.11 | SNMP's own statistics | snmpInPkts, snmpInBadCommunityNames, snmpEnableAuthenTraps |

Useful facts

- sysUpTime (TimeTicks) = time since the management part of the system was last re-initialised; used with counters to calculate rates and to detect restarts.

- sysObjectID identifies the vendor/model (an OID under enterprises).

- sysServices is a number encoding which layers the device operates at (value = sum of 2^(L-1) over layers L; e.g. router = 4 -> layer 3, host = 72 = layers 4 and 7).

- ifOperStatus: up(1), down(2), testing(3). ifAdminStatus is the desired state (read-write) and ifOperStatus is the actual state (read-only). Setting ifAdminStatus is how a manager brings an interface up or down.

- tcpConnTable is indexed by four values: local address, local port, remote address, remote port. Instance example: tcpConnState.10.1.1.1.80.10.1.1.9.1025.

## B6. The SNMP Communication Model

### B6.1 Administrative model (who may access what)

SNMPv1 security is very simple and is based on community.

- Community: a relationship between an agent and a set of managers, identified by a community name (string), e.g. 'public' or 'private'.

- Community profile: defines the MIB view (subset of objects visible) and the access mode (READ-ONLY or READ-WRITE) for that community.

- SNMP access policy: the pairing of a community with a community profile.

- Authentication: the community string is sent in clear text inside every message. Therefore SNMPv1 security is weak: anyone who sniffs the network can learn it.

- Authentication failure: the agent may send an authenticationFailure trap.

The agent checks (1) is the community name valid, (2) does the profile allow the requested object and the requested operation (access mode is combined with the object's own ACCESS).

Manager --> message(version, community, PDU) --> Agent
                                       |
                    community valid? -> MIB view includes object? -> access mode allows op? -> perform

### B6.2 Message format and PDUs

An SNMP message is a wrapper around a PDU:

+---------+-----------+------------------+
| Version | Community |   SNMP PDU       |
+---------+-----------+------------------+
Version = 0 for SNMPv1

It travels as the payload of a UDP datagram (agent port 161, manager trap port 162). An implementation must accept messages of at least 484 bytes.

Five SNMPv1 operations / PDUs

| PDU | Direction | Purpose |
| --- | --- | --- |
| GetRequest | Manager -> Agent | Read one or more object instances |
| GetNextRequest | Manager -> Agent | Read the next object in lexicographic order (used to walk MIB/tables) |
| SetRequest | Manager -> Agent | Write (change) object values |
| GetResponse | Agent -> Manager | Reply to Get, GetNext and Set (carries values and error info) |
| Trap | Agent -> Manager | Unsolicited alarm/event; not acknowledged |

Format of Get/GetNext/Set/GetResponse PDU

+-------------+------------+--------------+--------------+--------------------------------+
| PDU type    | request-id | error-status | error-index  | variable bindings (OID, value) |
+-------------+------------+--------------+--------------+--------------------------------+

- request-id: matches response to request (and detects duplicates).

- error-status: 0 noError, 1 tooBig, 2 noSuchName, 3 badValue, 4 readOnly, 5 genErr. Zero in requests.

- error-index: points to the first variable binding that caused the error.

- variable bindings (varbind list): pairs of object name and value (value is NULL in a get request).

Format of the Trap PDU (different!)

+-----------+------------+------------+--------------+---------------+------------+----------+
| PDU type  | enterprise | agent-addr | generic-trap | specific-trap | time-stamp | varbinds |
+-----------+------------+------------+--------------+---------------+------------+----------+

- enterprise: OID of the object generating the trap (sysObjectID).

- agent-addr: IP address of the agent.

- generic-trap (7 standard traps): 0 coldStart, 1 warmStart, 2 linkDown, 3 linkUp, 4 authenticationFailure, 5 egpNeighborLoss, 6 enterpriseSpecific.

- specific-trap: vendor-defined code, used only when generic-trap = 6.

- time-stamp: sysUpTime when the event occurred.

### B6.3 How each operation works

GetRequest: manager sends list of OIDs (instances, e.g. sysDescr.0). The agent returns a GetResponse with values. All-or-nothing: if any variable cannot be returned, no values are returned and error-status (noSuchName/tooBig/genErr) plus error-index is reported.

GetNextRequest: the agent returns the lexicographic successor of the OID given. Uses:

- Discover which objects exist (walk the whole MIB by repeatedly using the returned OID).

- Read a table when row indexes are unknown: the manager asks for the next of the column OID without index; the agent returns first row's instance; repeat. It ends when the response leaves the table (the next object belongs to another subtree) or 'noSuchName' at the end of the MIB.

SetRequest: modifies values. Errors: readOnly/noSuchName if object not writable, badValue for wrong type/value. Set is also atomic: either all variables are set or none. Used to control, e.g., ifAdminStatus.

Trap: sent by the agent on events (link up/down, restart, authentication failure). No response, so a lost trap is not detected.

Polling vs traps - and trap-directed polling

| Polling | Traps |
| --- | --- |
| Manager asks periodically | Agent reports when event occurs |
| Reliable detection but high traffic and delay | Instant and low traffic, but unreliable (UDP, no ack) |
| Better for performance data | Better for fault alarms |

Trap-directed polling: the manager receives a trap and then polls the agent for details - combining low traffic with reliability. This is the recommended SNMP practice.

Message-handling steps (agent side, for exam): parse ASN.1 message -> check version -> check community -> parse PDU -> check community profile for access -> execute operation on MIB -> build GetResponse -> send back via UDP to manager's address and port.

Exam Points: Draw the PDU formats; list 5 operations with direction; list 6 error-status values and 7 generic traps; explain why trap is unacknowledged; explain get-next on a table.

## B7. The Functional Model

The functional model describes the applications built on top of SNMP and how they cover FCAPS. SNMPv1 is mostly data-oriented: functions are implemented by the manager by reading/writing MIB objects.

| Function | How SNMP supports it |
| --- | --- |
| Operation / configuration | SetRequest on writable objects (ifAdminStatus, sysName, sysContact, ipForwarding); read configuration with Get |
| Fault | Traps (linkDown, coldStart), polling ifOperStatus, error counters (ifInErrors); test reachability |
| Performance | Counters/gauges (ifInOctets, ifOutQLen); RMON gives deeper data; compute utilisation and error rate |
| Security | Community strings, MIB views, access modes; authenticationFailure trap |
| Accounting | Interface octet counters per segment or per host (limited; better with RMON) |

A typical manager has MIB browser, graphing, polling engine, trap receiver, alarm display and a database - these are the application layer of the functional model.

Connecting the four models: the organization model says who talks, the information model says what is talked about (SMI, MIB), the communication model says how it is transported (PDUs, community), and the functional model says why (FCAPS applications). In one sentence: SNMP = SMI + MIB + protocol + community-based administration.

# PART C - UNIT II (b): SNMPv2 MANAGEMENT

## C1. Major Changes in SNMPv2

SNMPv1 had problems: weak security, inefficient retrieval of bulk data (one row at a time with get-next), no manager-to-manager communication, only 32-bit counters (a 100 Mb/s link wraps its octet counter in minutes), limited error reporting, awkward traps, and no standard way to create/delete table rows. SNMPv2 (1993) improved these.

Changes grouped by area

| Area | SNMPv1 | SNMPv2 |
| --- | --- | --- |
| Protocol operations | 5 PDUs | Added GetBulkRequest, InformRequest; Trap re-defined as SNMPv2-Trap; GetResponse renamed Response; Report (reserved, used by v3) |
| Data retrieval | One row per get-next | GetBulk returns many variables in one response |
| Manager-to-manager | None | InformRequest (acknowledged) between managers |
| Error handling | 6 error codes; get is all-or-none | 18 error codes; exception values per variable (noSuchObject, noSuchInstance, endOfMibView) so partial results are returned |
| SMI | Counter, Gauge 32-bit | Counter64, Integer32, Unsigned32, BITS; new macros and textual conventions |
| Status/access | mandatory/optional/obsolete; ACCESS | current/deprecated/obsolete; MAX-ACCESS (adds read-create) |
| Table handling | Not defined | RowStatus for row creation/deletion |
| Security | Community string only | Party-based (authentication MD5, privacy DES) - too complex, replaced by community-based SNMPv2c |
| Trap format | Special PDU | Same as other PDUs (varbind list); snmpTrapOID identifies the event |
| Set | Atomic, limited errors | Atomic, plus detailed errors (wrongType, wrongLength, inconsistentValue, ...) |

Variants: SNMPv2p (party-based, 1993 - the original), SNMPv2c (community-based - widely deployed), SNMPv2u (user-based security), SNMPv2* (not standardised). SNMPv3 later adopted user-based security (USM) and view-based access control (VACM).

## C2. SNMPv2 System Architecture

The SNMPv2 entity can act as manager, agent, or both. Manager-to-manager communication is added.

Manager-of-managers (NMS)
        |  (SNMPv2 inform / get / set)
   Mid-level manager  <-- dual role: agent upward, manager downward
        |
   Agents / Proxy agents (SNMPv1 devices)

Roles of an SNMPv2 entity: a manager role (generates requests, receives traps/informs), an agent role (responds, generates notifications). Entities with both roles are dual-role.

Administrative framework (party-based SNMPv2p)

| Concept | Meaning |
| --- | --- |
| SNMPv2 party | A virtual execution context defined by network address (transport), authentication protocol and key, privacy protocol and key. Messages are sent from a party to a party. |
| Context | A named collection of managed object instances (a MIB view accessible by one agent); one agent can have several contexts. |
| Access policy (ACL) | A rule: (requesting party, target party, context, privileges) - what operations the requester may do on that context. |

This gives finer access control than SNMPv1 community, but the configuration was heavy - one reason v2 was not widely adopted in its original form. SNMPv2c simply reverted to the v1 style (community string) while keeping all the new SMI and protocol features.

SNMPv2 message: version (1 for SNMPv2c), community, PDU. In SNMPv2p the message has a wrapper with authentication information: src party, dst party, context, then the PDU.

## C3. SNMPv2 Structure of Management Information (SMI)

Defined in RFC 1442 (updated by RFC 2578-2580 for SMIv2). New in SMIv2

1. New data types

| Type | Description |
| --- | --- |
| Integer32 | Signed 32-bit integer |
| Unsigned32 | Unsigned 32-bit (same as Gauge32 range; Gauge32 clips at max) |
| Counter32 | Same as v1 Counter |
| Counter64 | 64-bit counter, 0 to 2^64 - 1; for high-speed links; Counter64 is not available in SNMPv1 |
| Gauge32 | Same as v1 Gauge |
| TimeTicks | Hundredths of a second |
| BITS | Enumerated bit string (named bit flags) |
| IpAddress, OCTET STRING, OBJECT IDENTIFIER, Opaque | As before |

2. New and changed macros

- OBJECT-TYPE: adds UNITS, MAX-ACCESS (not-accessible, accessible-for-notify, read-only, read-write, read-create), STATUS (current, deprecated, obsolete), INDEX/AUGMENTS for tables, DEFVAL.

- NOTIFICATION-TYPE: defines notifications (traps/informs) properly, replacing the TRAP-TYPE macro.

- MODULE-IDENTITY: names and gives revision history for a MIB module.

- OBJECT-IDENTITY: describes an OID assignment.

- TEXTUAL-CONVENTION: defines a new, reusable type (e.g. DisplayString, TruthValue).

- Conformance macros: MODULE-COMPLIANCE, OBJECT-GROUP, NOTIFICATION-GROUP, AGENT-CAPABILITIES - define what an implementation must support.

3. Textual conventions (RFC 1443/2579) - give semantic meaning to basic types.

| Convention | Meaning |
| --- | --- |
| DisplayString | Printable ASCII, up to 255 chars |
| PhysAddress | MAC address (OCTET STRING) |
| MacAddress | 6-byte IEEE address |
| TruthValue | true(1), false(2) |
| TestAndIncr | Used for set coordination (serial number) |
| RowStatus | Controls creation/deletion of table rows |
| StorageType | other, volatile, nonVolatile, permanent, readOnly |
| TimeStamp, TimeInterval | Time values relative to sysUpTime |
| AutonomousType, InstancePointer | Pointers to OIDs/instances |

4. Row creation/deletion with RowStatus

RowStatus values: active(1), notInService(2), notReady(3), createAndGo(4), createAndWait(5), destroy(6).

- createAndGo(4): manager sets all columns and RowStatus in one SetRequest; agent creates the row and makes it active.

- createAndWait(5): agent creates the row but it stays notReady/notInService until the manager sets all needed columns and then sets RowStatus = active(1). Used when many set operations are needed.

- destroy(6): delete the row.

5. Table definition: a table row is identified using INDEX (or AUGMENTS to extend another table's row with new columns, sharing its index).

6. Name tree for SNMPv2: internet 1.3.6.1 has snmpV2 (6) = 1.3.6.1.6, with snmpDomains (1), snmpProxys (2), snmpModules (3). snmpMIB = snmpModules.1 = 1.3.6.1.6.3.1.

## C4. The SNMPv2 Management Information Base (MIB)

SNMPv2 MIB (RFC 1450/3418) keeps MIB-II but replaces system and snmp groups and adds new ones. All MIB-II tables are still used; the interfaces group is extended by ifXTable (mib-2.31) with 64-bit ifHCInOctets/ifHCOutOctets counters and ifHighSpeed so high-speed links can be monitored without wrap.

snmpMIB groups

| Group | Content |
| --- | --- |
| system (1.3.6.1.2.1.1) | Same as v1 plus sysORLastChange and sysORTable (which MIB modules the agent supports) |
| snmp (1.3.6.1.2.1.11) | Statistics: snmpInPkts, snmpInBadVersions, snmpInBadCommunityNames, snmpInASNParseErrs, snmpEnableAuthenTraps. Removed obsolete v1 PDU counters |
| snmpMIBObjects.snmpTrap | snmpTrapOID (identifies the notification), snmpTrapEnterprise |
| snmpMIBObjects.snmpSet | snmpSetSerialNo (TestAndIncr): an advisory lock so that several managers do not set the same objects at the same time |
| snmpMIBObjects.snmpTraps (notifications) | coldStart, warmStart, linkDown, linkUp, authenticationFailure, egpNeighborLoss |

Trap OID mapping: in v2 the trap is named by an OID (snmpTrapOID.0 = e.g. 1.3.6.1.6.3.1.1.5.3 for linkDown), where the last digit (1 to 6) corresponds to v1's generic trap + 1.

## C5. SNMPv2 Protocol

### C5.1 Operations

| PDU | Direction | Notes |
| --- | --- | --- |
| GetRequest | M -> A | As v1, but returns exception values per variable instead of failing all |
| GetNextRequest | M -> A | As v1 |
| GetBulkRequest | M -> A | New; efficient retrieval of large tables |
| SetRequest | M -> A | Atomic; detailed errors |
| Response | A -> M | Replaces GetResponse |
| InformRequest | M -> M | Acknowledged notification between managers |
| SNMPv2-Trap | A -> M | Unacknowledged notification; standard PDU format |

### C5.2 PDU formats

Get, GetNext, Set, Response, Inform, SNMPv2-Trap all share one format:

+----------+------------+--------------+-------------+--------------------+
| PDU type | request-id | error-status | error-index | variable bindings  |
+----------+------------+--------------+-------------+--------------------+

GetBulkRequest reuses the two fields differently:

+----------+------------+-----------------+------------------+--------------------+
| PDU type | request-id | non-repeaters   | max-repetitions  | variable bindings  |
+----------+------------+-----------------+------------------+--------------------+

Error-status (18 codes): 0 noError, 1 tooBig, 2 noSuchName (v1), 3 badValue (v1), 4 readOnly (v1), 5 genErr, 6 noAccess, 7 wrongType, 8 wrongLength, 9 wrongEncoding, 10 wrongValue, 11 noCreation, 12 inconsistentValue, 13 resourceUnavailable, 14 commitFailed, 15 undoFailed, 16 authorizationError, 17 notWritable, 18 inconsistentName.

Exception values (inside varbind value, for Get/GetNext/GetBulk responses):

- noSuchObject: the agent does not implement that object.

- noSuchInstance: the object exists but the instance does not (e.g. ifDescr.99).

- endOfMibView: no further object in lexicographic order (end of MIB view).

### C5.3 GetBulkRequest - how it works

The request has N = non-repeaters and M = max-repetitions and R = number of other variables (repeaters). For the first N variables the agent behaves like GetNext (one successor each). For the remaining R variables it performs up to M GetNext operations each, in an interleaved manner.

- Max number of variables in the response = N + (M x R).

- If the response would exceed message size, the agent returns as many as fit; endOfMibView is returned for lists that end early.

- The manager sets M large to pull a table quickly; N = 0 for pure table walks.

Example: non-repeaters N = 1, max-repetitions M = 2, request varbinds: sysUpTime, ifInOctets, ifOutOctets. Here R = 2. The response contains: sysUpTime.0 (non-repeater), then ifInOctets.1, ifOutOctets.1, ifInOctets.2, ifOutOctets.2. Total = 1 + 2x2 = 5 variables.

Advantages of GetBulk: fewer messages, less bandwidth and delay, fewer UDP round trips, and especially efficient over slow/long-delay links.

### C5.4 InformRequest and traps

- InformRequest is sent from one manager to another about a local event/information; the receiver sends back a Response with the same varbinds. Because it is acknowledged, the sender retransmits if no response arrives - reliable manager-to-manager notification.

- SNMPv2-Trap: the first two varbinds are always sysUpTime.0 and snmpTrapOID.0; further varbinds carry data. No acknowledgement.

### C5.5 Transport

Primary mapping is UDP (same ports 161/162); also defined for OSI CLNS/CONS, AppleTalk DDP, Novell IPX, so SNMPv2 can run over multiple protocols. Minimum message size supported is 484 bytes.

## C6. Compatibility with SNMPv1

SNMPv1 and SNMPv2 are not directly interoperable (different PDUs, different SMI, different data types), and many devices stay on v1 for years, so coexistence strategies are needed.

1. Bilingual (dual-protocol) manager: the manager speaks both v1 and v2. It keeps a record per agent (which version it uses), and sends the right message format to each. Simple, but the NMS is more complex.

2. Proxy agent: an SNMPv2 manager talks SNMPv2 to a proxy agent, which translates to SNMPv1 to the real v1 agent and back.

SNMPv2 Manager <--v2--> Proxy Agent <--v1--> SNMPv1 Agent

Proxy translation rules (examples):

- v2 GetBulk -> converted by proxy to a series of v1 GetNext requests.

- v1 Trap -> proxy converts to SNMPv2-Trap using snmpTrapOID mapping.

- v1 error noSuchName -> v2 exception noSuchObject (or noSuchInstance).

- v1 GetResponse -> v2 Response.

- Counter64 values cannot be sent to a v1 manager: the proxy drops/omits the variable (the v1 side returns noSuchName).

- v1 error codes not present in v2 are mapped to the nearest v2 code (e.g. genErr).

3. SMI conversion (MIB translation): a v1 MIB can be converted to SMIv2 (replace TRAP-TYPE with NOTIFICATION-TYPE, STATUS mandatory -> current, ACCESS -> MAX-ACCESS) by following guidelines.

SNMPv1 vs SNMPv2 - quick comparison table

| Feature | SNMPv1 | SNMPv2 |
| --- | --- | --- |
| RFCs | 1155, 1157, 1213 | 1441-1452 (v2p); 1901-1910 (v2c) |
| PDUs | 5 | 7 (GetBulk, Inform added) |
| Counters | 32-bit only | 32 and 64-bit |
| Bulk transfer | No | GetBulk |
| Manager-to-manager | No | InformRequest |
| Security | Community (clear text) | Party-based / community (v2c) |
| Error codes | 6 | 18 + exception values |
| Get behaviour | All-or-none | Partial results with exceptions |
| Trap | Special PDU format | Same PDU format, snmpTrapOID |
| Row creation | Undefined | RowStatus |
| Architecture | Manager-agent | Manager-agent-manager |